## 1. Table Overview

Table Name: opentrace-prod-5ga4.bronze.nakuru_air_quality_archive

What the Table Entails

This table contains raw hyper-local air quality measurements from low-cost sensors stationed in Nakuru, Kenya. It models particulate matter and environmental conditions as a high-frequency event log. For OpenTrace, this data is vital for assessing the impact of air quality on agricultural health and local climate conditions in the Rift Valley.

- Granularity: One row per Sensor per Timestamp (roughly every 2–3 minutes).

## 2. Source Details (Web API Specific)

- API Provider: Sensor.Community (or local OpenAQ/PurpleAir node)

- API Type: REST / CSV Archive Export

- Endpoint(s): /static/v1/data.json or Archive S3 Buckets

- HTTP Method: GET

- Authentication: None (Public Citizen Science Data)

- Pagination Strategy: Date-based file chunks

- Ingestion Frequency: Monthly (based on "source_archive" naming)

- Historical Coverage: Since 2021 (specific to the Nakuru sensor network)

## 3. Relationships & Dependencies

- Primary Key: Composite Key (sensor_id, timestamp)

- Foreign Keys: None (Primary sensor data).

- Relationship Type: One-to-Many (One sensor generates thousands of records).

- Join Notes: Use lat and lon to spatially join this data with fao_rl (Land Use) to determine if high particulate matter (pm10/pm2_5) correlates with specific agricultural or industrial land zones.

## 4. Column-Level Documentation

| Column Name | Description | Data Type | Format | Nullable | Example Value |
| --- | --- | --- | --- | --- | --- |
| sensor_id | Unique identifier for the hardware node | INTEGER | ID | No | 144 |
| sensor_type | Hardware model (e.g., SDS011 for particulates) | STRING | Text | No | SDS011 |
| location | Internal system location ID | INTEGER | ID | No | 3528 |
| lat / lon | WGS84 Decimal coordinates of the sensor | FLOAT | Decimal | No | -0.286 / 36.067 |
| timestamp | Precise time of the measurement | TIMESTAMP | ISO-8601 | No | 2021-08-03T07:20:18... |
| pm10 | Particulate matter < 10 micrometers | FLOAT | µg/m³ | Yes | 5.0 |
| pm2_5 | Particulate matter < 2.5 micrometers | FLOAT | µg/m³ | Yes | 4.0 |
| humidity_pct | Relative humidity | FLOAT | Percentage | Yes | null |
| temp_c | Ambient temperature | FLOAT | Celsius | Yes | null |
| source_archive | Metadata indicating the batch source | STRING | Text | No | August 2021 Archive |

### Notes & Edge Cases

- Data Gaps: Columns like humidity_pct and temp_c are null in this specific sensor (ID 144). The SDS011 sensor only measures particulates; temperature/humidity require a secondary sensor (like a DHT22) on the same node.

- Outlier Detection: Low-cost sensors are prone to "spikes" during high humidity. In the Silver layer, values for pm10 > 500 should be flagged as potential sensor errors.

- Timezones: The timestamp includes +00:00 (UTC). Since Nakuru is in EAT (UTC+3), a transformation will be helpful for diurnal (day/night) analysis.

- Duplicate Logic: Sensors occasionally double-post. A QUALIFY ROW_NUMBER() OVER(PARTITION BY sensor_id, timestamp ORDER BY ingested_at DESC) = 1 logic is recommended when moving to Silver.